|  |  |  |  | ZN8-53B5M213T-J1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Key Features: - GENSTAR IVS 2.0 - AI Analytics (Artificial Intelligence) - artificial intelligence using DeepLearning algorithms for instant and precise detection and recognition of objects such as persons and vehicles. - Smart Encode - dynamically adapts network stream and codec parameters to maintain recording continuity while minimizing archive storage requirements. - Image sensor schedules - image sensor operation can be configured based on time or Day/Night mode. - Corridor mode - adapts the field of view to scenes with a significant disproportion between the height and width of the monitored area. - Supports detection of faces/persons/vehicles - Supports P2P cloud - NDAA compliant - device compliant with the U.S. National Defense Authorization Act. |  |
| IP camera with GENSTAR IVS 2.0 analytics |  |  |  |  |  |
| Model | ZN8-53B5M213T-J1 |  |  |  |  |
| Image sensor | 1/3" progressive scan CMOS |  |  |  |  |
| Sensitivity (AGC ON) | Color: 0.01 lux @ F1.6 / IR ON: 0 lux |  |  |  |  |
| Day/Night mode | Automatic IR-cut filter |  |  |  |  |
| Electronic shutter | 1/5 - 1/20,000 |  |  |  |  |
| Iris | Fixed Iris |  |  |  |  |
| WDR mode | d-WDR |  |  |  |  |
| Noise reduction | 2D / 3D-DNR |  |  |  |  |
| Lens | Motorized zoom with AF: 2.7 - 13.5 mm, H: 90° - 29.8° |  |  |  |  |
| IR illuminator | 4 x IR LED, range up to 60 m, 850 nm |  |  |  |  |
| AV compression | H.265 ¦ H.264 ¦ MJPEG ¦ G.711U ¦ G.711A ¦ RAW |  |  |  |  |
| Bitrate (CBR/VBR) | Stream 1: 200 Kbps - 12 Mbps |  |  |  |  |
|  | Streams 2/3: 100 Kbps - 6/3 Mbps |  |  |  |  |
| Resolution and frame rate | 2560×1920, 2560x1440, 2304x1296 (25 fps) |  |  |  |  |
|  | 1920x1080 and 1280x720 (25 fps) |  |  | AI analytics capabilities: GENSTAR IVS 2.0 |  |
| Video streams | Stream 1: max. 2560x1920 (5MP) |  |  | AI engine features (Artificial Intelligence) | Object detection: DeepLearning engine |
|  | Stream 2: D1, VGA, CIF |  |  |  | Detection: face, person, vehicle. |
|  | Stream 3: VGA, CIF, QVGA |  |  |  | Object classification: person, vehicle (Detector) |
| Image settings | Rotation, saturation, brightness, contrast, sharpness |  |  |  | Simultaneous multi-object tracking (Tracker) |
| Digital functions | HLC ¦ BLC ¦ ROI ¦ DEFOG |  |  | Behavioral detection | Intrusion, loitering, wrong direction, illegal parking, people counting, |
| Corridor mode | Yes (9:16) |  |  |  | Single/double perimeter line crossing. |
| Privacy masks | 4 zones |  |  |  |  |
| Events | Motion detection (22x18 grid), |  |  |  |  |
|  | GENSTAR IVS 2.0 analytics, D/N switching, |  |  | Technical Specifications |  |
| Network protocols | IPv4/IPv6, HTTP/HTTPS, DNS, DDNS, DHCP, |  |  | Reset button | Yes |
|  | PPPoE, RTSP/ RTCP/RTP, TCP/UDP, NTP, ARP |  |  | SD card support | MicroSD card slot, max. 1 TB |
|  | UPnP, FTP, SMTP, QoS, ICMP, IGMP, SMTP |  |  | Power supply | 12VDC / PoE (802.3af / class-3) |
|  | 802.1x, SNMP, Multicast |  |  | Microphone | Yes, built-in |
| RTSP support | Standard RFC2326 (VLC Player / QuickTime) |  |  | Power consumption | 1.6 W (IR-LED OFF) / 5W (IR-LED ON) |
| Security | User authentication, hardware WatchDog |  |  | Power output | N/D |
| Compatibility | ONVIF (Profile S/T/G/M), SDK, CGI |  |  | Operating temperature | -30°C … +50°C |
| Ethernet interface | 10/100 Base-T, RJ45 |  |  | Max. humidity | 90% |
| Audio | N/D |  |  | Certifications | CE / FCC / IP66 |
| Alarm | N/D |  |  | Dimensions / weight | Ø 130 mm (base) x 113 mm / 720 g |
| Analog output | N/D |  |  | Adjustment range | 355° (H) / 75° (V) / 355° (sensor rotation) |

Security. OS / Firmware protection; Encrypted firmware, secure boot, signed firmware. User authentication; Digest authentication, brute-force attack protection. Network authentication; IEEE 802.1X (EAP-TLS). Secure communication; HTTPS, WSS (WebSocket Secure), SRTP. Access control; IP-based access control, automatic logout. Data protection; Credential encryption. Secure data storage; SD card partition encryption using AES-256 and hardware Secure Element. Audit; Access / system / event log management. Supply chain security; SBOM. Built-in GSTAR IVS 2.0 video analytics (AI Analytics) AI multi-object; Intrusion; Smart detection. Line crossing; Double line; Group loitering. Wrong direction; Illegal parking; People counting. DORI Distance. Lens; Detection; Observation; Recognition; Identification. 2.8mm; 82 m; 32.8 m; 16.4 m; 7.2 m. OODPCVS Distance (OODPCVS distances calculated based on pixel density thresholds according to IEC/EN 62676-4:2025.). General overview; Object outline; Object discrimination.; Behavior perception; Object characterization; Verification; Detailed analysis. 117.5 – 342.5 m; 58.8 – 171.3 m; 29.4 – 85.6 m; 18.8 – 54.8 m; 9.4 – 27.4 m; 4.7 – 13.7 m; 1.6 – 4.6 m.. Mounting adapters:; ZA8-CBK627B only in combination with ZA8-JBMP-2. ZA8-CBK656A; ZA8-CBK627B. Compatibility:. GENSTAR NVR; CORTROL VMS; NDAA Compliance.

| Dimensions: |  |  |
| --- | --- | --- |